PLANNING PARAMETERS FOR

TERRESTRIAL DIGITAL SOUND

BROASDCASTING SYSTEMS IN VHF

BANDS

HD RADIO SYSTEM

Brazil, May 2013.

This document have been written by Flávio Ferreira Lima from Ministry of Communication in Brazil and revised by Russ Mundschenk from iBiquity Digital Corporation in United States of America.

Sumário

Introduction	4

1.	Receiver noise input power level	4

2.	Minimum receiver input power level	5

3.	Effective antenna aperture	6

4.	Minimum power flux-density	6

5.	Calculation of minimum RMS	7

6.	Calculation of minimum median RMS field-strength level	7

7.	Referências	8

# Introduction

This document describes some parameters that will be very important to close the planning parameters for terrestrial digital sound broadcasting systems in HDRadio.

## Receiver noise input power level

The thermal noise level for HDRadio can be calculated with the formula:

Pn=F+10*log10(k*T*B)

Where,

Pn -> Receiver noise input power level.

F -> Noise Figure.

k -> Boltzmann constant: k = 1.3806504* 10-23 Ws/K.

T -> Temperature in Kelvin, T= 290◦.

B -> Bandwidth in Hz.

Considering the information at Tabela 1, it is possible to calculate the Receiver noise input power level for each kind of the receiver. For example, the calculation for Portable Indoor Handset receiver:

Pn=F+10*log10(k*T*B)

Pn=25+10*log10(1.3806504*10-23*290*100000)

Pn=-128,98 dBW

Tabela 1 – Receiver Noise Figure.

| Reception Mode | FX | MO | PO | PI | PO-H | PI-H |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna type | External fixed | Adapted | External telescopic / ear bud | External telescopic / ear bud | Internal | Internal |
| Receiver System Noise Figure, [dB] | 7 | 7 | 8 | 8 | 25 | 25 |

## Minimum receiver input power level

To determine the Minimum receiver input power level for HDRadio can be use the following formula:

Ps,mindBW=CNmin+Pn+Li2

Where,

Ps,min -> Minimum receiver input power level.

C/N -> Carrier / Noise.

Pn -> Receiver noise input power level.

Li -> Implementation loss factor.

RUSS: in this point I have doubt about the values for C/N in HD Radio. In page 22 until 24, referent to tables 4-3 to 4-7, there are the values in Cd/No [dB-Hz]. In my point of view this parameters are different in relation to C/N. On the other hand, in page 30 Appendix C, we have IBOC FM Conversion of Cd/No to Digital CNR or SNR Example. In this way am I right to say that C/N is like SNRdB for HDRadio? If not, please could you tell me how can calculate it? Below I am sending you an example, if I am right.

Example: considering the table 4-1 in reception mode MO for service MP1, we could calcite the C/N as:

CNdB=SNR=CdN0dB-48.45 dB

CNdB=SNR=57.2-48.45=8.75 dB

Does it correct?

## Effective antenna aperture

To determine the Effective antenna aperture for HDRadio, considering the central operate frequency at 100 MHz, it can be use the following formula:

AadBm2=10*log101.644π300fMHz2+GDdB4

Where,

Aa -> Effective antenna aperture (dBm2).

GD -> Antenna gain in dBd.

Example: considering the table 3.10 in reception mode MO, the antenna gain correction is 0 dB, then the effective antenna aperture is:

AadBm2=10*log101.644π3001002+0

AadBm2=0.698765

## Minimum power flux-density

Minimum power flux density is the magnitude of the Poynting vector at receiving place. To determine the Minimum power flux density for HDRadio can be use the following formula:

φmindBW/m2=Ps,mindBW-AadBm2+LfdB3

Where,

min -> Minimum power flux density.

Ps,min -> Minimum receiver input power level.

Aa -> Effective antenna aperture (dBm2).

Lf -> feeder loss (dB).

Example: considering a reception mode MO, the minimum power flux density is:

φmindBW/m2=-135.23-0.698765+0.3

φmindBW/m2=-135.63

## Calculation of minimum RMS

To determine the minimum RMS field-strength level at the location of the receiving antenna, it is possible to use the following equation:

EmindBμV/m=φmindBW/m2+10log10ZF0dBΩ+20log101V1μ

With:

ZF0=2μ0ε0≅120π, the characteristic impedance in free space

Resulting in:

EmindBμV/m=φmindBW/m2+145.8dBΩ

Example: considering a reception mode MO, the minimum RMS is:

EmindBμV/m=-135.63 +145.8

EmindBμV/m=10.17

## Calculation of minimum median RMS field-strength level

For the different receiving scenarios the minimum median RMS field strength is calculated as follows:

- For fixed reception:

  - Emed=Emin+Pmm+Cl

Where:

Emin -> Minimum RMS.

Pm,m -> Allowance for manmade noise.

Cl -> Location correction factor.

- For portable outdoor and mobile reception:

  - Emed=Emin+Pmm+Cl+Lh

Where:

Emin -> Minimum RMS.

Pm,m -> Allowance for manmade noise.

Cl -> Location correction factor.

Lh -> Antenna height loss.

- For portable indoor reception:

  - Emed=Emin+Pmm+Cl+Lh+Lb

Where:

Emin -> Minimum RMS.

Pm,m -> Allowance for manmade noise.

Cl -> Location correction factor.

Lh -> Antenna height loss.

Lb -> Building penetration loss.

## Referências

[1] Recommendation ITU-R BS.1660-6, “Technical basis for planning of terrestrial digital sound broadcasting in the VHF band”, 2012.

[2] EBU-TECH 3317, “Planning parameters for hand held reception concerning the use of DVB-H and T-DMB in Bands III, IV, V and the 1.5 GHz band”, 2007.

[3] GE06 , “Final Acts of the Regional Radiocommunication Conference for planning of the digital terrestrial broadcasting service in parts of Regions 1 and 3, in the frequency bands 174-230 MHz and 470-862 MHz (RRC-06) Annex3: Technical basis and characteristics”.

[4] Recommendation ITU-R BS.599, “Directivity of antennas for the reception of sound broadcasting in band 8 (VHF)”, 1982.

[5] ETSI TR 101 190, “Digital Video Broadcasting (DVB); Implementation guidelines for DVB terrestrial services; Transmission aspects”.

Document: HD RadioTM System Planning Parameters for VHF Band II, table 4.4 at page 20.